Methods for digitizing contour plots, adding Lidar data, and calculating DEM’s of difference

Note: If something doesn’t work. Save Arc. Close arc. Open arc. Magic!

- Scanning Alger Contour plots into ARC

  - Adding map document to Arc found at http://courses.umass.edu/nrc592g-cschweik/pdfs/Georeferencing_a_map.pdf

  - Copy image (Contour plot from Alger) into paint.net and save as XX_Alger.jpg

  - Start ArcMap

    - Customize  extensions  check “3d Analyst, ArcScan, and Spatial Analyst”

    - Customize  Toolbars  Add ArcScan, Georeferencing , and Editor

    - File  Map document properties  check “Store relative pathnames to data sources”

    - Catalog  Connect to folder  “Thesis”

  - Setting jpg properties and adding to map

    - Folder Connections  .jpg file  right click  properties  Spatial Reference  Edit  WGS 1984 UTM 58 S  Apply

    - Add Data  jpg file  Build pyramids (Yes)

  - Georeferencing Contour map

    - Click “Add control points” on georeferencing toolbar

    - Zoom in to BM1

    - Click once on BM1 then again to turn crosshair red from green

    - Repeat for all BM’s

    - Once all BM’s are marked, open excel file “1993_Survey_All_Transects” and note lat/long of all BM’s

    - Open “Link Table”(Sometimes located just off edge of screen…)  Replace X_Map and Y_Map with BM’s lat/long  Zoom to full extent to go to true map location

    - In georeferencing toolbar, click “Rectify”  Change save location to proper folder  Save saves as XX_Alger1.tiff file with “1” added to end of name

    - Close Map Doc  start blank map  Click “+” (Expand tiff file) button to new tiff file  Add 1 layer (Band_1) of rectified tiff to new map

    - In Table of Contents, right click Tiff file  Properties  Symbology  Classified  Classes = “2”  “OK”

- Creating Vector data from contour lines in Alger Scans

  - Information found at http://www.fas.harvard.edu/~chinaarch/Downloads/gis/L05_creatingvector/L05_creatingvector%20English.pdf

  - Creating a new shapefile in arc catalog

    - In Catalog, right click folder that contour plot is in  New  Shapefile

      - Name with NO SPACES similar to “XX_Shp.shp”

      - Feature Type = “PolyLine”

      - Coordinate System = “58 S”

      - “OK”

    - Adding Attributes

      - Right Click new shapefile in Catalog!  Properties  “Fields” Tab  Below ID in Field Name type “Elevation”  under Data Type select “Double”  “OK”

  - Creating Vectors

    - In Editing Toolbar select “Start Editing”, Then select the Shapefile in “Create Features”, and then “line”

    - In ArcScan toolbar select “Raster Snapping Options” button  Check for correct values (What are they? 20,5,200,0)  “OK” Make sure arcscan is up and other features to make vectorization work.

    - Under the “Vectorization” Menu, select “Vectorization Settings”  “Styles”  “Contours”  “OK”

    - Next to the Vectorization Menu, select “Vectorization Trace” Tool.

    - Click on beginning of Contour Line, and allow arrow to trace line automatically

      - Sometimes at intersections or other unnatural features, the line stops. “Jump” Over the break with “s” and continue on with line

      - Double Click or press F2 to end Vector trace

      - Arc loves to not work after crossing a line. It jumps giant gaps to connect with completely unrelated lines although it should follow the line indicated. Need to contact Arc about this, or just finish the contour manually.

      - If there are large gaps in the contour lines, manually change the classification scheme to account for gray areas to be black and not white.

    - ArcScan allows for automation of contours, but I don’t believe this works very well. Instructions in website or pdf above.

    - Once line is created, go into Attribute Table of shapefile and add elevation of contour line just created

      - Check BM elevation from Alger

      - Check BM elevation from UNAVCO

      - +/- difference from the two for each contour elevation

    - Repeat for all contour lines

    - “Stop Editing”

    - Save Map Frequently as ex. “XX.mxd”

- Topo to raster 3D analyst for now? Spatial does stream polyline better(Not using this anymore)

  - Input feature data = XX_Shp.shp

  - Field = Elevation

  - Rename and save to proper folder! “XX_T2R”

  - Cell size=? Make everything 0.1 (meters) but takes forever with Lidar

  - After finishing, can change the symbology to full 32 layers instead of ~9.

    - Formatting

      - Need to figure out how to label Orange, Green, and Black mats so can tell differences between mats and seasons.

  - Water Surface Elevations. Maybe?

  - Rocks

  - Other important features

- Cropping out extra raster (Not using this either anymore)

  - Need to crop out extra raster data because is not correct or accurate

    - Make new shapefile (XX_Clp.shp)

      - Polygon, and same coordinate system

    - Start Editing, select XX_Clp

    - Trace perimeter of contour shapefile. Make sure end snapping is selected.

    - Clip(Data Management)

      - Select Raster to clip  XX_T2R

      - Output extent  XX_Clp

      - Select “Use input features…”

      - Rename output  XX_T2R_Clp

      - May need to change symbology from stretched to classified. This asks to compute histogram. Select yes. Then import data from XX_T2R for same color scheme.

- Importing LiDAR data into ARC

  - If Lidar data is single file continue to b. Otherwise follow below.

    - Open up a new command window in dos. Click start and type cmd

    - Navigate to folder containing lidar sets using the command “cd foldername” to go up one folder at a time or “cd..” to go back a folder. Ex. C:\Users\Crisp\Desktop\Thesis\Antarctic Scans\Lidar\F2

    - Type “copy /b filename1 + filename2 + filename3 outfilename

    - Concatenated file should appear in a few seconds!

  - In Arc, Under “File”  Add data  Add XY data (This is very important XY data!!!)

    - Select LiDAR set

    - Choose corresponding X, Y, and Z columns for data

    - Update Coordinate system to 58S

    - Hit OK

      - Once the addition is finished, it will bog down ARC adding each point. Unselect the Lidar data in the table of contents and proceed to point to raster below

  - Point to Raster in conversion tools

    - This is going to take a while so be prepared

    - Choose newly imported XY data set

    - Value field is Z or elevation

    - Save file as XX_P2R_YYYY Unfortunately Arc is omnipotent and gets angry when there are more than 13 characters. Assumption is that cellsize (Below) is 0.1m

    - Cellsize = 0.1

      - This means each cell with be 0.1 m wide. You can change this to a smaller dimension, but it will take exponentially longer.

    - You can get into detail with cell assignment and priority fields, but typically leave them at default

    - Wait for 35-45 hours (min) for this to finish.

    - Once completed, change symbology to from stretched to classified

      - Symbology adoption however has problems.

      - Ex. Lidar has range of 19-31 meters. Adopted contour plot has range of 25-28.

      - Therefore everything on Lidar below 25 or above 28 is same color.

      - Fix? Find the best fit scan/plot and correlate everything else to that piece

  - Patching holes in raster and adding hillshade

    - “Raster Calculator” in Spatial Analyst

      - Enter code  Con(IsNull("Input_Raster"),FocalStatistics("Input_Raster",NbrRectangle(3, 3),'MEAN'),"Input_Raster")  Output Raster = “XX_Patch”  OK

      - The input raster is the lidar DEM that has holes in it either due to water or shadows from rocks or poor TLS placement

      - The “3” is the width and height of the patching

      - Explanation: The above code finds cells within the DEM that are NoData and takes a 3X3 matrix surrounding it, computes the mean, and then outputs that elevation value

    - “Hillshade” Spatial Analyst

      - Input Raster = XX_Patch  Output Raster = XX_YYYY_HS  azimuth altitude and z are optional to change  OK

      - Need grayscale

      - http://resources.arcgis.com/en/help/main/10.1/index.html#//009t0000007q000000

    - Represent stream within channel at certain flows?

      - ArcHydro

    - Make a model builder!

- Adding LAS data to Arc Scene

  - LASTools

    - Open Txt2Las in the lastools/bin folder

    - Click Browse  \.. to go back folder. Search for RGB lidar file you want

    - Output  Proper directory  Name file XX_YYYY_LAS

    - More attributes  Select R, G, and B

    - CHANGE FORMAT FROM LAZ TO LAS!!! Have no idea why this isn’t automatic

    - Hit run!  takes ~30 seconds.

  - Importing LAS file

    - Open new arcscene map and save as XX.sxd

    - Navigate to folder being used for stream in folder connections

    - Right click stream folder  new  LAS Dataset  name “XX.lasd”

    - Right click XX.lasd  Properties  XY coordinate system tab = 58S  LAS Files tab  Add files  select newly created las file from lastools  OK

    - Drag XX.lasd into map

    - In the LAS Dataset Layer toolbar select the symbolize button (Button with bunch of dots) and select RGB.

    - Now have lidar map to fly through!

- GCD – Geomorphic Change Detection

  - Download and Install

    - Download and install GCD 6 from http://gcd.joewheaton.org/downloads

    - Go to Arc  Customize  Add in manager and GCD should automatically be added

    - In toolbars select GCD and pin it to the toolbar

  - Building DEMS

- Adding XY Algae Data to Alger Scans

  - Algae Data

    - Formatting

      - Into .csv’s

        - Data should already be organized in one large spreadsheet based off of flow season

        - Create 1 csv file for 1 stream for 1 season

          - Ex. Bowles_1213.csv, Bowles_1314.csv…

          - .csv files should have 4 columns

            - Easting

            - Northing

            - Orthometric height

            - Description

    - Adding XY data

      - Find .csv file  Create feature class  From XY table

      - Change x and y and coordinate system

      - Save as default name

      - This creates a shapefile within the stream folder and not the default Arc folder which might be better for editing and finding etc.

      - Add new shp file to arc map

  - Stream Boundary

    - Create new polygon shapefile called “Stream_Boundary”

    - Start editing

    - Follow stream path and save edits

  - Contour Polyline to point

    - Feature Vertices to Points

      - Input Features = “XX_Shp”

      - Output = “Contour_Points”

- Point differencing from base lidar

  - G = 1, B = 2, O = 3, R = 4

  - Extract Values to points

    - Input point features “XYXX_YYYY”

    - Input Raster “Patch”

      - If not doing patch, the NoData cells indicate water. Can be used for algae analysis later on in different study

    - Name “YYYY_E2P”

      - YYYY = year

      - E2P = Extract to points

    - Interpolate points Check

  - Clip(Analysis)

    - Input features = YYYY_E2P

    - Clip Features = Stream_Boundary

    - Output = YYYY_Clip

  - Erase

    - Input features = YYYY_E2P

    - Erase Features = Stream_Boundary

    - Output = YYYY_Erase

  - Export feature attribute to ASCII – Find in toolboxes not custom box

    - Input feature = YYYY_clip/erase

    - Value field = Orthometri, RASTERVALU,  ( + Point_Desc for Clp file!)

    - Delimiter = space

    - Output = YYYY_Ers/Clp

    - Field names = Unchecked!

  - Contour Clip and Erase

    - Perform same as above except export the contour season as 9293 so it plots next to 9394 or the season that it was created. Maybe figure a better way to plot this?